# Agent Brief: Upgrade aco-ekf-soc-estimation to an industry-grade BMS project

Repo: github.com/ByteBeast-1/aco-ekf-soc-estimation. Give this whole document to the coding agent. Work phase by phase, one branch per phase, small commits.

## 0. Rules for the agent (read first)

- Do not break what works. simulink_matlab/run_bms_regression.m (33 checks) and the Simulink-vs-library back-to-back test (max difference about 4e-12 %) must still pass after every phase.

- simulink_matlab/lib/*.m is the single source of truth for algorithms and parameters. Never paste algorithm code into .slx files. Models are regenerated by build_bms_models.m; edit the builder, not the binary.

- Never invent numbers. Every figure in a README or report must come from a script that can be re-run. If MATLAB, a toolbox, a compiler or a dataset is not available in your environment, write the code and the tests, mark the item NOT VERIFIED and say exactly what must be run on a machine that has it.

- Keep existing public function names and signatures unless a task says otherwise. Add, do not rename.

- Every new feature needs a test (MATLAB regression check, pytest, or C test) and a line in the changelog.

## 1. Current state (facts found in the repo)

- advanced_bms_aco_ekf.slx: simulated 2-RC Arrhenius plant (Battery_Physics_Plant), seeded faultable sensors (Sensor_Measurement), one estimator (ACO_EKF_Controller, mode 0), 8 scopes. Init: bms_setup('synthetic').

- sota_bms_aco_ekf.slx: replays real NASA B0005/6/7 data (NASA_Replay) through three estimators side by side: EKF_Baseline (mode 0), SOTA_Adaptive_Controller (mode 2, Sage-Husa R) and SOH_Capacity_Estimator (mode 10, capacity state plus Sage-Husa R). Init: bms_setup('nasa').

- Both call lib/bms_est_step.m. It is a 4-state Joseph-form EKF with states [SOC, V1, V2, capacity ratio]; the capacity state is frozen unless mode bit 8 is set. Mode bits: 1 = RLS on R0, 2 = Sage-Husa R, 4 = Sage-Husa Q, 8 = capacity state.

- ACO: lib/bms_aco_tune.m (ACO_R, 4 log-parameters: Q_soc, Q_v1, Q_v2, R) writes lib/bms_tuned_data.m. Tuning data is disjoint from evaluation data.

- C twin: c_firmware/bms_ekf.c/.h, mode 0 only, float32, 236 bytes of state. Software-in-the-loop (SIL) check exists in lib/bms_sil_run.m.

- Known real-data results (from results/bms_regression_results.csv): fresh NASA cells MAE 0.66 % and 0.93 %; aged cells 2.3 to 4.9 %; Coulomb counting 6 to 12 %.

## 2. Phase 1: Make the documentation honest and consistent (do this first, about 1 day)

Problem: the root README claims MAE below 0.01 % and R^2 0.9999. Those numbers come from the Python pipeline on synthetic data and contradict the real-data regression.

Tasks:

- Rewrite README.md so the headline results table is generated from results/bms_regression_results.csv (script: tools/make_results_table.py, output pasted by the script into the README between marker comments).

- Split results into two clearly labelled tables: "Model-matched synthetic" and "Real NASA data (unseen cells)". State that synthetic numbers are optimistic.

- Fix wrong statements: "3-state" (it is 4-state with an optional capacity state), "2-subsystem" Simulink (it has plant, sensors and controller), and any place that calls data/NASA_B0005.csv real. That CSV is synthetic; real data is data_raw/*.mat. Either delete the synthetic CSV and its readers or rename it SYNTHETIC_B0005.csv.

- Rename "SOTA" in user-facing text to "Adaptive EKF" unless a published state-of-the-art method is actually reproduced and compared (see Phase 3).

- Remove duplicates: the notebooks exist both at the repo root and in docs/, as .md and .docx. Keep one copy of each under docs/, delete generated .docx from git (generate on demand), and update links.

- Add LICENSE (ask the owner which; default MIT), CHANGELOG.md, CONTRIBUTING.md, and a one-page architecture diagram in docs/.

- Fix requirements.txt with pinned versions (it is only 40 bytes now) and add environment.yml or pyproject.toml.

Acceptance: a new reader can find one README, one results table, and every number traces to a script.

## 3. Phase 2: Connect the two Simulink models (the main request)

Goal: one model family that can run any input source through any estimator set, with the same metrics code. Implement options A to D below; A and B are mandatory.

A. Common scenario and metrics layer (MATLAB, no Simulink change needed)

- Create simulink_matlab/bms_scenario.m: returns a struct with fields name, source ('synthetic'|'nasa'), I, V_ref, T, soc_ref, soc0, init_err, seed, fault_mode, params_variant.

- Create simulink_matlab/run_all.m: loops over a scenario list, runs both models (via Simulink.SimulationInput for sweeps), collects bms_log, calls lib/bms_metrics.m, and writes one CSV plus one comparison figure per scenario into results/.

- Add regression suite 8: the unified run must reproduce the existing per-model results within 1e-9.

B. Unified model bms_unified.slx (generated by build_bms_models.m)

- Input source as a Variant Subsystem, selected by workspace variable BMS_Source: SIM (plant plus sensors, from advanced model) or REPLAY (NASA data, from sota model).

- Estimator bank as a Variant Subsystem or an enabled set of three referenced estimators: EKF (mode 0), ADAPTIVE (mode 2), SOH (mode 10). Selected by BMS_EstimatorSet.

- Wrap the estimator as a referenced model bms_estimator_ref.slx so both legacy models and the unified model use the same block. Keep the old advanced_bms_aco_ekf.slx and sota_bms_aco_ekf.slx generated from the same builder, so nothing existing breaks.

- Common sensors: allow the REPLAY source to pass through Sensor_Measurement with fault injection too (flag BMS_InjectFaultsOnReplay), so real data can be tested under faults.

- Keep the bus objects and bms_data.sldd as the shared interface. Add a BMS_Reference bus (soc_ref, source tag) so the SIM and REPLAY sources expose the same reference signal.

C. Cross-feeding test matrix

- Run the three sota estimators on the advanced plant (ground truth, faults, Monte Carlo).

- Run the advanced ACO-EKF on the NASA replay.

- Produce a table: rows = estimator, columns = scenario, cells = MAE, max error, convergence time, fault-detect latency.

D. Single entry point

- bms_cli.m or run_everything.m: build -> regression -> run_all -> report. One command, exit non-zero on failure (needed for CI later).

Acceptance: run_all produces one table covering both source types and all three estimators; the legacy models still open and match the library.

## 4. Phase 3: Strengthen the science

- Prove the ACO contribution. Add lib/bms_tune_compare.m comparing, at the same evaluation budget (same number of fitness calls, many seeds): hand-tuned, random search, grid search, Nelder-Mead or fminsearch, particle swarm or genetic algorithm (MATLAB ga/particleswarm if available), and ACO_R. Report mean and spread of final fitness and of held-out MAE. If ACO does not win, say so; document the real finding.

- Reproduce one published baseline (for example the base paper's ACO-EKF as described, plus a standard adaptive EKF such as AEKF with innovation-based covariance matching) before using the word "SOTA". Cite the source in the doc.

- Multi-objective properly. The tuner currently scalarises MSE plus lambda times convergence time. Add a true Pareto front (NSGA-II style or MO-ACO with archive) and plot MSE vs convergence time; keep the scalar result as a default.

- Fix the OCV model. The synthetic polynomial reaches 4.59 V at 100 % SOC. Replace with a monotonic lookup table (griddedInterpolant/interp1 friendly, codegen-safe) calibrated per chemistry, with SOC range 0 to 1 mapping to 3.0 to 4.2 V for the NMC/NCA cells. Keep the old polynomial selectable as legacy.

- Aging and SOH. Improve capacity tracking: joint capacity and R0 estimation with sufficient excitation gating, or a dual-EKF (fast SOC filter, slow parameter filter). Report capacity-ratio error versus cycle number on all 3 NASA cells. Document the observability limit of one constant-current discharge.

- Detect the silent fault. Current-sensor offset raises max error from 0.40 % to 0.75 % without any flag. Add an offset detector (for example Coulomb-vs-voltage drift monitor at rest, or an extended state for current bias), with a detection flag bit 32.

- Observability and consistency checks. Add NEES and NIS whiteness tests (autocorrelation of innovation) as automated pass/fail criteria in the regression.

- Temperature. Test at 0, 10, 25, 40 degrees C. Add Arrhenius parameter identification from data instead of the fixed Ea of 18000 J/mol.

- Statistics. Report mean, standard deviation, 95th percentile and max error over at least 100 Monte Carlo runs, with confidence intervals.

## 5. Phase 4: Real and dynamic data

The biggest industry gap is that NASA data is a constant 2 A room-temperature discharge.

- Add loaders (lib/bms_load_<dataset>.m and Python equivalents) for at least one dynamic dataset: CALCE (INR 18650-20R / A123 with DST, FUDS, US06), Oxford Battery Degradation, Panasonic 18650PF (McMaster, with temperatures from -20 to 25 degrees C), or Samsung 30Q. Put download scripts in data/download_*.py, never commit large raw data; add checksums.

- Calibrate a separate cell parameter set per dataset (OCV from low-rate test, RC from pulse or ID script). Hold out whole cells and whole cycles for validation.

- Add scenarios: DST/FUDS/US06/UDDS/WLTP, low-temperature, aged cell, wrong initial SOC (plus and minus 20 %), sensor dropout.

- Update the results table with a "dynamic data" section.

## 6. Phase 5: Real BMS functions (system level)

- Protection and fault state machine (Stateflow, generated by the builder): states NORMAL, WARNING, DERATE, FAULT, SHUTDOWN. Inputs: voltage, current, temperature, estimator fault code, NIS flag. Define thresholds in bms_params. Add hysteresis and debounce timers.

- State of Power (SOP) estimate: maximum charge and discharge current limits from voltage and SOC limits using the 2-RC model.

- Pack model: series string (for example 4 to 96 cells), cell-to-cell spread in capacity, R0 and initial SOC, with per-cell estimators and a pack SOC definition.

- Cell balancing: passive balancing logic (bleed resistor model, voltage-threshold and SOC-based strategies) and optionally active balancing; metrics: time to balance, energy lost.

- Thermal model: lumped cell thermal model (heat from I^2 R, convection), coupled to the Arrhenius parameters. Feed estimated temperature, not only measured.

- Charging control: CC-CV charging with SOC-based termination and temperature derating.

## 7. Phase 6: Embedded and code generation

- Extend c_firmware/bms_ekf.c to all modes (adaptive R, RLS R0, capacity state) behind compile-time flags, keeping mode 0 as the minimal build. Re-run SIL for each mode against bms_est_step.m.

- Compile warnings clean with gcc -std=c99 -O2 -Wall -Wextra -Wconversion -pedantic, plus cppcheck --addon=misra (or clang-tidy) with a report committed under results/.

- Add unit tests (Unity or plain asserts) for: init, step, NaN/Inf inputs, glitch rejection, saturation limits, repeatability. Add Makefile or CMake.

- Fixed-point study: Q15 or Q31 version of the EKF; compare error and range against float32; document overflow analysis.

- Embedded Coder (if licensed): ert.tlc generation from the estimator model, equivalence test against the handwritten C (back-to-back), static analysis of generated code. If not licensed, write the config script and mark NOT VERIFIED.

- Target measurements: execution time and stack on a real MCU (STM32 or ESP32 or similar), or at minimum a cycle-accurate estimate with a QEMU or vendor simulator. Report time per step and RAM/flash.

- Optional PIL (processor-in-the-loop) harness reusing sil/sil_main.c.

## 8. Phase 7: Testing, CI and engineering process

- GitHub Actions in .github/workflows/: (a) Python lint (ruff), type check, pytest on the pipeline with a small fast dataset; (b) C build with gcc, run unit tests and the SIL smoke test, cppcheck; (c) optional MATLAB job using matlab-actions/setup-matlab and run-command for run_bms_regression (note licensing).

- Python tests (tests/): EKF vs reference values, ACO reproducibility with fixed seed, battery model sanity (energy, SOC bounds), data loader checks.

- Simulink Test (if available): test cases for each fault mode and each source, with pass criteria (MAE limits, fault flags, no NaN/Inf).

- Requirements traceability: docs/requirements.md with IDs (for example REQ-SOC-001: MAE <= X % on unseen real cells), each linked to a test and a result. Fill X from measured results plus a margin, not from wishes.

- Model quality: run Model Advisor / MAAB checks if available and commit the report; otherwise add the Simulink diagnostics already enabled (dangling ports, algebraic loops, bus mismatch) as a regression check.

- Coverage: record statement and decision coverage for the MATLAB library and C code (gcov).

- Repo hygiene: .gitattributes for .slx/.mat, move the 3 .mat files to Git LFS or a download script if size is a problem, add .editorconfig, pre-commit hooks, semantic version tags and GitHub Releases for milestones.

- Safety documentation (ISO 26262-style, educational level): hazard list (wrong SOC leads to over-discharge, thermal event), safety goals, fault coverage table mapped to the diagnostic bits, and known limitations.

## 9. Phase 8: Deliverables

- A results report (docs/Results_Report.md plus a PDF export) generated from the CSVs, with figures: SOC tracking, error histograms, NIS, fault-injection timelines, Pareto front, aging trend.

- A short demo: a script that launches the unified model on one real and one simulated scenario and plots live (animate_bms_live.m already exists; wire it to the unified model).

- A 5-minute presentation (update docs/presentation/) built from the report, with the honest numbers.

- A release tag v1.0 once all acceptance criteria below pass.

## 10. Suggested order and effort

| Order | Phase | Why first |
| --- | --- | --- |
| 1 | Phase 1 (honest docs) | Cheapest, fixes credibility |
| 2 | Phase 2 (connect models) | The main request, enables everything else |
| 3 | Phase 3 items 1, 6, 7 (ACO proof, silent fault, consistency tests) | Strengthens the core claim |
| 4 | Phase 7 (CI, tests) | Protects all later work |
| 5 | Phase 4 (dynamic data) | Biggest realism gain |
| 6 | Phase 6 (embedded) | Industry signal |
| 7 | Phase 5 (pack and BMS functions) | Scope expansion |
| 8 | Phase 8 (report) | Packaging |

## 11. Definition of done (acceptance criteria)

- All README numbers are generated by scripts and match the CSVs.

- run_bms_regression passes, including the new unified-model suite, and CI is green.

- One command reproduces the main results table.

- ACO is compared with at least three alternative tuners over multiple seeds, with the result stated honestly.

- At least one dynamic, multi-temperature dataset is validated with held-out cells.

- The C firmware passes SIL equivalence for every supported mode, compiles warning-free, and has a static-analysis report.

- Every requirement has an ID, a test and a measured result; anything unverified is labelled NOT VERIFIED.

- Known limitations are listed in the README, not hidden.